# Flood Guard Barrier Annual Inspection & Readiness Checklist

Complete once in January and again during the full deployment drill in May.

| January inspection date: | ____________________________ | Inspector(s): | ____________________________ |
| --- | --- | --- | --- |
| May drill date: | ____________________________ | Inspector(s): | ____________________________ |

## A. Barrier Panels, Uprights & Seals

| Inspection item | JAN | MAY | Notes / initials |
| --- | --- | --- | --- |
| ☐ All barrier panels are present for each of the three protected doors/openings. | ☐ | ☐ |  |
| ☐ Every panel is clearly labeled with the correct door location and installation order/position. | ☐ | ☐ |  |
| ☐ Panels are straight and free of cracks, dents, warping, corrosion or other damage. | ☐ | ☐ |  |
| ☐ Rubber gaskets and seals are present, flexible, firmly attached and free of cuts, splits, flattening or dry rot. | ☐ | ☐ |  |
| ☐ Gasket surfaces are clean and free of sand, grit, paint, caulk or other debris. | ☐ | ☐ |  |
| ☐ Uprights/channels are secure, straight, clean and free of corrosion or damage. | ☐ | ☐ |  |
| ☐ Caulk around the uprights/channels is continuous, bonded and free of gaps, cracks or separation. | ☐ | ☐ |  |
| ☐ Door thresholds and all barrier contact surfaces are clean, smooth and unobstructed. | ☐ | ☐ |  |
| ☐ Flex Seal tape is on hand, in usable condition and sufficient for temporary emergency sealing. | ☐ | ☐ |  |

## B. Bolts, Clamps & Installation Hardware

| Inspection item | JAN | MAY | Notes / initials |
| --- | --- | --- | --- |
| ☐ All bolts, threaded rods, fasteners and pressure clamps are present for all three doors. | ☐ | ☐ |  |
| ☐ Bolts and threaded parts are clean, lightly greased and turn freely by hand. | ☐ | ☐ |  |
| ☐ Pressure clamps operate smoothly, tighten fully and show no cracks, bends or excessive wear. | ☐ | ☐ |  |
| ☐ No hardware is rusted, stripped, cross-threaded, bent or missing. | ☐ | ☐ |  |
| ☐ Each door has a separate, clearly labeled container/bag containing its installation parts. | ☐ | ☐ |  |
| ☐ Spare fasteners and commonly needed replacement parts are available and labeled. | ☐ | ☐ |  |
| ☐ All hardware containers close securely and are stored with the correct barrier components. | ☐ | ☐ |  |

## C. Tools, Supplies & Safety Equipment

| Inspection item | JAN | MAY | Notes / initials |
| --- | --- | --- | --- |
| ☐ An ample number of correct-size Allen wrenches is available, including spares. | ☐ | ☐ |  |
| ☐ Compressed air is available and charged/operational for cleaning channels and surfaces. | ☐ | ☐ |  |
| ☐ A step stool is present, stable and in safe working condition. | ☐ | ☐ |  |
| ☐ Cleaning supplies are available: brushes, rags, broom, dustpan and/or shop vacuum. | ☐ | ☐ |  |
| ☐ Lubricant/grease and cleaning materials are available for bolts and threaded hardware. | ☐ | ☐ |  |
| ☐ No tool, key, code or critical item is controlled by only one person. | ☐ | ☐ |  |

## D. Instructions, Labels & Records

| Inspection item | JAN | MAY | Notes / initials |
| --- | --- | --- | --- |
| ☐ The Flood Guard manufacturer’s installation instructions are complete, legible and stored with the system. | ☐ | ☐ |  |
| ☐ The Lodge Flood Barrier Deployment Information Booklet is current, complete and easy to locate. | ☐ | ☐ |  |
| ☐ A printed copy of the Lodge Flood Guard Barrier Readiness and Deployment Program is stored at the Lodge. | ☐ | ☐ |  |
| ☐ Electronic copies of the instructions and program are available to the Worshipful Master, Wardens and Secretary. | ☐ | ☐ |  |
| ☐ Door photographs/diagrams clearly show panel order, upright location, clamp location and finished installation. | ☐ | ☐ |  |
| ☐ All containers, panels, tools and storage locations are clearly labeled. | ☐ | ☐ |  |
| ☐ Previous drill/deployment records and photographs are retained with Lodge records. | ☐ | ☐ |  |
| ☐ Any changes made after prior drills or deployments have been added to the instructions. | ☐ | ☐ |  |

## E. Response Team & Access Readiness

| Inspection item | JAN | MAY | Notes / initials |
| --- | --- | --- | --- |
| ☐ The Worshipful Master has appointed the Deployment Coordinator, Assistant Coordinator and Second Alternate. | ☐ | ☐ |  |
| ☐ The Secretary/Communications Officer has a current phone, text and email roster for all team members. | ☐ | ☐ |  |
| ☐ At least six trained members are identified, including a minimum four-person deployment crew and alternates. | ☐ | ☐ |  |
| ☐ At least two people are trained for every critical deployment responsibility. | ☐ | ☐ |  |
| ☐ At least three officers have the Lodge door code and know how to access the building. | ☐ | ☐ |  |
| ☐ Every team member has acknowledged his role and availability for the upcoming storm season. | ☐ | ☐ |  |
| ☐ The Slack Storm Preparedness Channel and “Lodge Storm Response” group-text procedure are understood. | ☐ | ☐ |  |
| ☐ Automatic transfer-of-command procedures are reviewed and understood. | ☐ | ☐ |  |

## F. Full Functional Test & Storage

| Inspection item | JAN | MAY | Notes / initials |
| --- | --- | --- | --- |
| ☐ All components have been removed from storage and inventoried. | ☐ | ☐ |  |
| ☐ Every protected opening has been completely installed using the manufacturer’s instructions. | ☐ | ☐ |  |
| ☐ No member worked alone during the test installation. | ☐ | ☐ |  |
| ☐ Each panel, seal, bolt and pressure clamp fit and operated correctly. | ☐ | ☐ |  |
| ☐ Each completed barrier was inspected for properly seated seals and full opening coverage. | ☐ | ☐ |  |
| ☐ Photographs were taken of each fully installed barrier for future reference. | ☐ | ☐ |  |
| ☐ Total deployment time was recorded: __________ hours __________ minutes. | ☐ | ☐ |  |
| ☐ All problems, missing parts and training needs were documented and assigned for correction. | ☐ | ☐ |  |
| ☐ After the test, components were removed, cleaned, dried and returned to their labeled storage locations. | ☐ | ☐ |  |
| ☐ All repairs/replacements were completed or assigned with a responsible person and due date. | ☐ | ☐ |  |

## Corrective Action Follow-Up

| Issue / missing item | Assigned to | Due date | Completed / verified |
| --- | --- | --- | --- |
|  |  |  | ☐ |
|  |  |  | ☐ |
|  |  |  | ☐ |
|  |  |  | ☐ |
|  |  |  | ☐ |

## Annual Readiness Sign-Off

| Annual readiness certification: | ☐ January review complete ☐ May drill complete |
| --- | --- |
| Deployment Coordinator: | ____________________________________________ |
| Equipment/Building Committee: | ____________________________________________ |
| Secretary filing date: | ____________________________ |

Additional notes / follow-up:

# Flood Guard Barrier Coordinator Deployment Procedure Checklist

Use for the January procedure review, the May full deployment drill, and any actual storm deployment.

| Use: | ☐ January review ☐ May drill ☐ Actual deployment | Coordinator: | ________________________ |
| --- | --- | --- | --- |
| Date: | ________________________ | Start time: | __________ |
| Weather/event: | ________________________ | Crew count: | __________ |

## 1. Activation, Authority & Safety Decision

| Deployment step | Done | Notes / initials |
| --- | --- | --- |
| ☐ Confirm the current readiness level and the official trigger for action. | ☐ |  |
| ☐ Review National Hurricane Center, Pinellas County Emergency Management and Alert Pinellas information. | ☐ |  |
| ☐ Confirm that roads, bridges, weather and evacuation timing allow safe deployment. | ☐ |  |
| ☐ Target completion approximately 48 hours before hazardous conditions, when forecasts permit. | ☐ |  |
| ☐ Do not delay until a hurricane warning if delay may make access unsafe. | ☐ |  |
| ☐ Confirm who is in command: Deployment Coordinator, Assistant Coordinator or Second Alternate. | ☐ |  |
| ☐ Apply the no-assumption rule: responsibility remains active until deployment is scheduled, completed or formally cancelled. | ☐ |  |

## 2. Crew Callout & Communication

| Deployment step | Done | Notes / initials |
| --- | --- | --- |
| ☐ Start or activate the “Lodge Storm Response” group text. | ☐ |  |
| ☐ Send the deployment callout with date, time, Lodge arrival time and assigned crew. | ☐ |  |
| ☐ Require each crew member to reply YES or NO within 30 minutes. | ☐ |  |
| ☐ Call any crew member who has not acknowledged within 60 minutes. | ☐ |  |
| ☐ Activate alternates immediately when needed. | ☐ |  |
| ☐ Confirm a minimum four-person deployment crew; no person will work alone. | ☐ |  |
| ☐ Notify Lodge officers of the deployment schedule and next communication time. | ☐ |  |

## 3. Before Leaving for the Lodge

| Deployment step | Done | Notes / initials |
| --- | --- | --- |
| ☐ Confirm Lodge door code/key access with at least two participating members. | ☐ |  |
| ☐ Confirm barriers, labeled hardware containers, tools and instructions are at the Lodge. | ☐ |  |
| ☐ Bring charged phones, flashlights, gloves and weather-appropriate clothing. | ☐ |  |
| ☐ Confirm compressed air and cleaning supplies are available. | ☐ |  |
| ☐ Confirm step stool and all required Allen wrenches are available. | ☐ |  |
| ☐ Advise all participants not to travel if conditions become unsafe. | ☐ |  |

## 4. Arrival & Work Assignment

| Deployment step | Done | Notes / initials |
| --- | --- | --- |
| ☐ Coordinator conducts a brief safety and installation review before work begins. | ☐ |  |
| ☐ Record arrival/start time and names of all participants. | ☐ |  |
| ☐ Assign members by doorway/opening or by specific component/task. | ☐ |  |
| ☐ Place the correct labeled panels and hardware container at each of the three doors. | ☐ |  |
| ☐ Review the manufacturer’s instructions and door-specific photographs/diagrams. | ☐ |  |
| ☐ Confirm that no member is assigned to work alone. | ☐ |  |

## 5. Prepare Each Protected Opening

| Deployment step | Done | Notes / initials |
| --- | --- | --- |
| ☐ Inspect the door/opening, uprights, channels, threshold and surrounding caulk for damage. | ☐ |  |
| ☐ Remove dirt, sand, leaves and debris from channels, thresholds and gasket contact surfaces. | ☐ |  |
| ☐ Use compressed air, brush, rags or vacuum as needed. | ☐ |  |
| ☐ Confirm channels are clear and panel labels match the opening. | ☐ |  |
| ☐ Inspect gaskets for damage and confirm they are clean and properly positioned. | ☐ |  |
| ☐ Apply temporary Flex Seal tape only where needed and appropriate for emergency sealing. | ☐ |  |
| ☐ Confirm bolts, threaded rods and clamps are clean, lightly greased and operating freely. | ☐ |  |

## 6. Install the Barrier System

| Deployment step | Done | Notes / initials |
| --- | --- | --- |
| ☐ Follow the manufacturer’s instructions exactly for each opening. | ☐ |  |
| ☐ Install panels in the labeled order and correct orientation. | ☐ |  |
| ☐ Install all bolts, fasteners and pressure clamps for that opening. | ☐ |  |
| ☐ Tighten evenly so the barrier seats squarely without twisting or damaging seals. | ☐ |  |
| ☐ Confirm rubber gaskets remain properly seated and continuous across contact surfaces. | ☐ |  |
| ☐ Confirm all clamps are secure and all bolts are fully engaged. | ☐ |  |
| ☐ Repeat the process for all three protected doors/openings. | ☐ |  |

## 7. Coordinator’s Final Inspection

| Deployment step | Done | Notes / initials |
| --- | --- | --- |
| ☐ Inspect every protected opening personally. | ☐ |  |
| ☐ Confirm every required panel and component is installed. | ☐ |  |
| ☐ Confirm seals appear properly seated with no visible gaps. | ☐ |  |
| ☐ Confirm all bolts and pressure clamps are secure. | ☐ |  |
| ☐ Confirm all doors/openings included in the system are protected. | ☐ |  |
| ☐ Check the building for any remaining flood vulnerabilities. | ☐ |  |
| ☐ Confirm no tools, hardware, containers or loose materials remain outside. | ☐ |  |
| ☐ Take clear photographs of each completed barrier and the overall secured building. | ☐ |  |

## 8. Secure Building & Confirm Completion

| Deployment step | Done | Notes / initials |
| --- | --- | --- |
| ☐ Return unused tools and spare parts to a secure indoor location. | ☐ |  |
| ☐ Confirm doors, interior areas and the building are secured. | ☐ |  |
| ☐ Record completion time: __________ Total deployment time: __________. | ☐ |  |
| ☐ Confirm every participating member has safely departed or is departing together. | ☐ |  |
| ☐ Send the standard “DEPLOYMENT COMPLETE” message with date and time. | ☐ |  |
| ☐ Retain photographs, crew list, times and notes with Lodge records. | ☐ |  |

## 9. Drill Removal / Post-Storm Removal

| Deployment step | Done | Notes / initials |
| --- | --- | --- |
| ☐ Confirm conditions are safe before removing any barrier components. | ☐ |  |
| ☐ Photograph any damage, water intrusion or unusual condition before removal. | ☐ |  |
| ☐ Remove panels and hardware carefully using the manufacturer’s instructions. | ☐ |  |
| ☐ Clean and dry panels, gaskets, bolts, clamps, channels and tools. | ☐ |  |
| ☐ Inspect for damage, corrosion, deterioration or missing parts. | ☐ |  |
| ☐ Lightly grease clean bolts/threaded parts before storage. | ☐ |  |
| ☐ Return each door’s parts to its clearly labeled container. | ☐ |  |
| ☐ Return all panels, tools, instructions and supplies to their designated storage locations. | ☐ |  |
| ☐ Correct or assign all missing/damaged items immediately. | ☐ |  |

## 10. After-Action Review

| Deployment step | Done | Notes / initials |
| --- | --- | --- |
| ☐ Within 30 days, review what worked properly and what caused delay or confusion. | ☐ |  |
| ☐ Confirm whether enough trained members responded. | ☐ |  |
| ☐ Review whether all equipment, communications and access procedures worked. | ☐ |  |
| ☐ Determine whether deployment occurred early enough. | ☐ |  |
| ☐ Document required repairs, replacements, training changes or instruction updates. | ☐ |  |
| ☐ Present recommended changes to the Worshipful Master and Secretary. | ☐ |  |
| ☐ Retain the completed checklist with the Lodge’s annual drill/deployment records. | ☐ |  |

## Final Sign-Off

| Coordinator certification: | ☐ All required steps completed and verified |
| --- | --- |
| Coordinator name/signature: | ____________________________________________ |
| Secretary/record keeper: | ____________________________________________ |
| Checklist and photographs filed on: | ____________________________ |

Prepared from the Gulf Beach Lodge Flood Guard Barrier Readiness and Deployment Program. Keep this checklist with the manufacturer’s installation instructions and deployment information booklet.